NTU SPACE DATA CENTER

NTU SPACE DYNAMICS

Software Module Design Report

Orbit • Attitude • Solar Environment • Power and Thermal • Access Analysis

| Institute | College of Computing and Data Science, Nanyang Technological University |
| --- | --- |
| Author | Qin Jiahao |
| Reviewer | Liu Xiyuan |
| Issue date | 17 August 2026 |

College of Computing and Data Science, Nanyang Technological University

Version Control

| Abstract | This report defines the architecture, algorithms, public interfaces, data contracts, verification approach, and deployment constraints of the NTU Space Dynamics Python module. |  |  |  |
| --- | --- | --- | --- | --- |
| Keywords | digital twin; orbit dynamics; attitude dynamics; Sun environment; access analysis; verification |  |  |  |
| Change Record |  |  |  |  |
| Revision | Change Request | Date | Author | Description |
| 1.0 | Initial issue | 2026-08-17 | Qin Jiahao | Orbit dynamics module test report based on the retained outline. |

- TABLE OF CONTENTS

1 Document Overview	1

1.1 Purpose	1

1.2 Scope	1

1.3 Intended readers	1

1.4 Normative terminology	1

2 Requirements and Design Objectives	2

3 Overall Design	3

3.1 Layered architecture	3

3.2 Package decomposition	3

3.3 Shared data flow	4

4 Concise Orbital-Dynamics Background	4

4.1 Reference Frames	4

4.2 Reference Frames Transformation	6

4.3 Orbit propagation	8

4.4 Classical orbital elements	8

4.5 Non-spherical gravity	9

4.6 Sun's celestial position	9

4.7 Solar Radiation Pressure	10

4.8 Power and Thermal	11

4.9 Access Calculation Using Cubic Hermite Interpolation	12

5 Detailed Functional Design	15

5.1 Common data models	15

5.2 Orbit propagation	15

5.2.1 Two-body propagator	15

5.2.2 J2 propagator	16

5.2.3 SGP4 propagator	16

5.2.4 High-precision propagator	16

5.3 High-order Earth gravity	16

5.4 Element and Cartesian-state conversion	17

5.5 Coordinate and frame transformations	17

5.6 Attitude conversion and dynamics	18

5.7 Solar environment and radiation pressure	18

5.8 Power considering panel temperature	18

5.9 Access propagation and interpolation	19

6 Public Interface Design	19

6.1 General contracts	19

6.2 Primary calling interfaces	20

6.3 Minimal end-to-end call	20

7 End-to-End Operational Workflow	21

8 Numerical Design and Performance	23

9 Verification and Validation	23

10 Deployment and Configuration	24

10.1 Runtime	24

10.2 Installation and VS Code interpreter	24

11 Limitations and Planned Evolution	24

12 Requirements Traceability Matrix	26

Document Overview

Purpose

The purpose of this report is to provide an implementable and reviewable design baseline for the current NTU Space Dynamics software module. It explains what the module computes, how the major algorithms are composed, which frames and units are accepted, and what each public interface returns.

Scope

- Orbit-state and classical-element conversion, analytical and numerical orbit propagation, and TLE-based SGP4 propagation.

- Static high-order Earth gravity from bundled EGM2008 or external fully normalized ICGEM/GRACE coefficient files.

- GCRS, ITRS, and TEME state transformation, plus WGS-84 geodetic/ECEF conversion.

- Euler-angle conversion, quaternion rigid-body dynamics, Sun position and beta-angle analysis.

- Conical eclipse, solar irradiance, cannonball solar-radiation pressure, and one-node solar-panel thermal power.

- Ground-target access from direct orbit propagation or Hermite interpolation of sampled states.

- End-to-end computation and diagnostic plotting of position, velocity, beta angle, illumination, power, temperature, quaternion, and body-rate histories.

Intended readers

The report is written for spacecraft simulation developers, mission-analysis engineers, software integrators, verification personnel, and reviewers who need enough orbital-dynamics context to evaluate the design without requiring a full astrodynamics textbook treatment.

Normative terminology

Table 3  Terminology and frame conventions

| Term | Meaning in this module |
| --- | --- |
| GCRS | Geocentric Celestial Reference System; the numerical-propagation inertial-frame contract. |
| ITRS / ECEF | Earth-fixed frame used for gravity synthesis and ground geometry. |
| TEME | True Equator, Mean Equinox frame returned by the SGP4 implementation. |
| Osculating elements | Instantaneous Keplerian elements derived from a Cartesian state. |
| Mean elements | Averaged orbital parameters represented by a TLE and interpreted by SGP4. |
| HPOP | Composable high-precision Cowell propagation architecture; not a single undeclared fidelity level. |
| Access | Time interval satisfying Earth-clear line of sight, minimum elevation, sensor FOV, and optional range. |

Requirements and Design Objectives

The software was designed around explicit model semantics and auditability. A calculation result must identify its time system, units, frame, force-model selection, gravity data, and numerical tolerances. The module therefore favors a small set of composable Python interfaces over an opaque monolithic simulation service.

Table 4  Principal design objectives

| ID | Objective | Design response |
| --- | --- | --- |
| DO-01 | Complete requested functional coverage | Public APIs cover all requested orbit, attitude, solar, power, pressure, gravity, and access functions. |
| DO-02 | Safe data contracts | SI units, aware UTC datetimes, and explicit GCRS/ITRS/TEME labels are enforced at public boundaries. |
| DO-03 | Fidelity is selectable | Two-body, J2, SGP4, and HPOP represent distinct model semantics instead of a single hidden propagator. |
| DO-04 | Reproducibility | EGM2008 provenance and SHA-256 are documented; IERS, ephemeris, truncation, and tolerance settings are task inputs. |
| DO-05 | Offline operation | Astropy built-in ephemeris and locally available IERS tables support deterministic disconnected runs. |
| DO-06 | Extensibility | HPOP accepts custom force models and gravity coefficients; torque can be constant or callable. |
| DO-07 | Verification | Twenty-three unit tests cover invariants, reference vectors, transforms, gravity synthesis, solar geometry, thermal trends, and access events. |

Design rule: A frame label is part of the numerical value. A TEME vector must never be renamed GCRS without a real transformation, and a velocity transform must include transport terms caused by Earth rotation.

Overall Design

Layered architecture

The module is organized as a layered scientific library. User applications and batch workflows call analysis services. Those services reuse common types, frame transformations, gravity synthesis, force models, integration, interpolation, and event-search utilities. External scientific libraries are isolated behind explicit frame and model contracts.

Figure 1  Layered software architecture

Package decomposition

Table 5  Package-level decomposition

| Package or module | Responsibility | Principal public API |
| --- | --- | --- |
| types.py | Validated immutable data contracts | OrbitState, Ephemeris, ClassicalElements, AttitudeEphemeris |
| orbits/ | Element conversion and propagation | oe_to_rv, rv_to_oe, TwoBodyPropagator, J2Propagator, SGP4Propagator, HighPrecisionPropagator |
| gravity.py | ICGEM loading, Earth orientation, spherical harmonics | GravityFieldCoefficients, EarthOrientationCache, SphericalHarmonicGravity |
| coordinates.py | Celestial/terrestrial transformations | eci_to_ecef, ecef_to_eci, transform_ephemeris, geodetic/ECEF conversion |
| attitude.py | Euler conversion and rigid-body propagation | ypr_to_euler, euler_to_dcm, EulerDynamics |
| solar.py | Sun, eclipse, beta angle, irradiance, SRP | sun_position_analytical, sun_position_ephemeris, eclipse_fraction, sun_intensity, sun_radiation_pressure |
| power.py | One-node panel thermal and electrical model | SolarPanel, power_considering_thermal |
| access.py | Ground target/sensor geometry and event refinement | GroundTarget, NadirSensor, access_propagation, access_interpolation |

Shared data flow

Figure 2  End-to-end computation flow

Orbit outputs are reused rather than re-derived by each subsystem. The same epoch sequence drives Sun vectors, eclipse, panel irradiance, attitude propagation, and access analysis. This alignment prevents hidden interpolation and frame mismatches between otherwise independent calculations.

Concise Orbital-Dynamics Background

Reference Frames

The orbit-dynamics module uses three reference frames to separate inertial orbit calculations, Earth-fixed geometry, and SGP4-compatible state representation. GCRS is used for numerical orbit states, ITRS for Earth-fixed gravity and targets, and TEME for raw SGP4 results.

The Geocentric Celestial Reference System (GCRS) is the primary frame for numerical orbit states. Its origin is located at the Earth’s center of mass, while its axes remain kinematically non-rotating relative to the celestial reference system. Spacecraft position and velocity, numerical propagation results, and Sun and Moon direction vectors are represented in GCRS. Each state record contains an epoch, a three-dimensional position vector, a velocity vector, units, and an explicit frame identifier.

The True Equator, Mean Equinox (TEME) frame is retained for compatibility with the SGP4 propagator and Two-Line Element data. Its equatorial plane follows the true equator of date, while its reference direction is associated with the mean equinox of date. Raw SGP4 position and velocity outputs are expressed in TEME and remain in this frame until an explicit conversion is performed. TEME states must therefore be transformed into GCRS for comparison with numerical orbit states or into ITRS for ground-access calculations.

The International Terrestrial Reference System (ITRS) is an Earth-fixed frame that rotates with the Earth. Its \(z\)-axis points toward the IERS Reference Pole, and its x-axis passes through the IERS Reference Meridian. The y-axis completes the right-handed coordinate system. Earth gravity coefficients, ground stations, surface targets, and geodetic longitude, latitude, and altitude are naturally defined in this frame. ITRS is therefore used to calculate ground access, elevation, azimuth, range, Earth-fixed illumination, and geographical ground tracks.

Table 1 Category and application of reference frames

| Reference Frame | Category | Primary Applications |
| --- | --- | --- |
| GCRS | Earth-centered inertial reference frame | Numerical orbit integration, Sun and Moon directions, and perturbation-force calculations |
| TEME | Earth-centered inertial-type reference frame based on a legacy approximation | Representation of raw SGP4 position and velocity outputs |
| ITRS | Earth-centered, Earth-fixed reference frame | Ground stations, ground targets, Earth gravity fields, subsatellite points, and access calculations |

The GCRS-to-ITRS position transformation can be expressed as

where  accounts for precession and nutation,  represents Earth rotation, and  represents polar motion. This transformation requires Earth-orientation parameters, including UT1–UTC and polar-motion coordinates. The corresponding velocity transformation must also include the derivative of the rotation matrix

where

Reference Frames Transformation

A coordinate transformation changes the numerical components of a vector without changing the represented physical position or direction. For two Cartesian frames \(A\) and \(B\), the position transformation is

where  is the direction cosine matrix from frame \(A\) to frame \(B\). For orthonormal reference frames,

When the destination frame rotates relative to the source frame, the velocity transformation must include the time derivative of the rotation matrix:

The second term is the transport term caused by frame rotation. Omitting this term produces incorrect Earth-fixed velocities, even when the transformed positions are correct.

A physical vector, such as gravitational acceleration evaluated at one instant, only requires a rotation of its components:

Transport terms are required when transforming coordinate derivatives, rather than an independently evaluated physical vector.

GCRS and ITRS transformation: The Geocentric Celestial Reference System (GCRS) is used for numerical orbit states, while the International Terrestrial Reference System (ITRS) is used for Earth-fixed geometry. The transformation can be expressed as

where  represents the celestial-to-intermediate transformation, including precession, nutation, and celestial-pole motion,  represents Earth rotation, evaluated using UT1, and  represents polar motion between the terrestrial intermediate frame and ITRS. The complete transformation therefore depends on the observation epoch, the adopted precession-nutation model, UT1–UTC, polar-motion coordinates, and leap-second data.

TEME transformations: The True Equator, Mean Equinox (TEME) frame is used as the native output frame of SGP4. The project first parses the user-supplied TLE and propagates it with the Vallado SGP4 implementation:

TEME is a legacy, date-dependent reference frame associated with the true equator and mean equinox. Its axes are not identical to either GCRS or ITRS. Consequently, a TEME state cannot be converted by changing its frame label.

When an SGP4 result is compared with a numerical orbit, the required transformation is

When an SGP4 result is used for ground-track or access calculations, the required transformation is

WGS-84 geodetic and ITRS coordinates: Ground stations and ground targets are specified by geodetic latitude , longitude, and altitude . These quantities are converted to ITRS/ECEF Cartesian coordinates using the WGS-84 reference ellipsoid. The first eccentricity squared and prime-vertical radius are

where   is the WGS-84 equatorial radius and  is the ellipsoid flattening. The Cartesian coordinates are

MOD-to-GCRS solar transformation: The analytical solar-position model initially produces the Earth-to-Sun vector in the mean equator and mean equinox of date frame (MOD). This vector must be converted before combination with GCRS spacecraft states:

The project represents the analytical vector with Astropy’s PrecessedGeocentric frame. Astropy then applies the required precession and frame transformation to produce a GCRS-compatible solar vector.

VVLH-to-GCRS transformation: The access module defines sensor directions in a local vertical, local horizontal frame using the project’s VVLH convention. The local basis is constructed from the GCRS position and velocity:

Here,  points toward nadir and  follows the orbit normal. A sensor boresight defined in VVLH is transformed into GCRS as

The resulting direction is compared with the GCRS target line of sight to calculate the sensor off-boresight angle.

Orbit propagation

An orbit is represented by position r and velocity v at an aware UTC epoch. Numerical propagation integrates the first-order state equations. The central term produces ideal Keplerian motion; each additional acceleration represents a declared perturbation.

Two-body propagation keeps only the central term. J2 adds the dominant oblateness effect. HPOP integrates a configurable sum of high-order gravity, third-body gravity, drag, solar pressure, relativity, and custom forces. SGP4 is different: it propagates the mean elements encoded in a TLE using the SGP4 analytical theory and returns TEME states.

Classical orbital elements

For elliptic motion the six classical elements are semi-major axis a, eccentricity e, inclination i, right ascension of the ascending node Ω, argument of periapsis ω, and true anomaly ν. They provide a compact geometric description, but Ω and ω become singular for equatorial and circular orbits. The implementation uses deterministic conventions so that OE→RV→OE→RV reconstructs the original state.

Figure 2 Demonstration of the classical orbital elements

Non-spherical gravity

Earth's mass distribution is represented by normalized spherical-harmonic coefficients  and . J2 is the dominant degree-two zonal contribution. Higher degrees and orders represent progressively finer latitude/longitude structure. Gravity synthesis is performed in ITRS; the acceleration is then rotated back to GCRS for integration.

The packaged EGM2008 subset is fully normalized and tide-free through degree/order 120. The HPOP default truncation is 70×70. Increasing degree improves spatial detail but increases computation cost approximately with the number of retained coefficients.

Sun's celestial position

At each requested UTC epoch, the numerical Sun-position path converts time to TT/TDB and evaluates the barycentric positions of the Sun and Earth from the selected ephemeris: the offline Astropy solution or a configured JPL binary kernel. The apparent geocentric vector is formed with a light-time correction, so the solar state is evaluated at the retarded epoch.

The corrected vector is normalized in GCRS. Right ascension and declination are obtained from its Cartesian components; the same unit vector is rotated to ITRS with the precession-nutation, Earth-rotation, and polar-motion matrices when Earth-fixed illumination or access geometry is required.

Model fidelity is controlled by the ephemeris source, leap-second data, and Earth-orientation parameters rather than by a propagation step size. The offline solution supports deterministic routine analyses; a version-controlled JPL kernel is selected when higher accuracy and reproducibility are required. Each result records the epoch, reference frame, ephemeris source, and kernel version.

Solar Radiation Pressure

Solar Radiation Pressure (SRP) is produced when solar photons transfer momentum to the spacecraft surface. The resulting force generally points away from the Sun. Its magnitude depends on the solar irradiance, spacecraft optical properties, effective illuminated area, spacecraft mass, Sun–spacecraft distance, and eclipse condition. Let  and  denote the positions of the Sun and spacecraft in the same inertial reference frame. The vector from the Sun to the spacecraft is , . The corresponding unit vector is

Because this vector points from the Sun toward the spacecraft, it also represents the direction of the SRP acceleration. Solar irradiance decreases according to the inverse-square law

where  is the solar irradiance at one astronomical unit. The corresponding radiation pressure is

where

A common orbit-dynamics approximation is the cannonball model, in which the spacecraft is represented by a body with a constant reference area and a single radiation-pressure coefficient. The SRP force is

and the corresponding acceleration is

where  is the illumination or visible-Sun fraction, is the radiation-pressure coefficient, is the reference illuminated area, m is the spacecraft mass, and  is the area-to-mass ratio. A perfectly absorbing surface has, while reflective surfaces have. The theoretical value approaches 2 for ideal specular reflection.

Power and Thermal

The incident solar power is calculated from the illuminated panel area, local solar irradiance, and solar incidence angle:

The factor  prevents the rear side of the panel from generating power. The irradiance  should already include Sun–spacecraft distance scaling and the applicable eclipse factor. Therefore,  becomes zero when the panel is shadowed or faces away from the Sun. The electrical conversion efficiency varies with panel temperature according to a linear temperature model:

The generated electrical power is then

Here,  denotes the conversion efficiency at the reference temperature . The coefficient  is normally negative because photovoltaic efficiency decreases as the cell temperature increases. In the numerical implementation, the calculated efficiency should be constrained to the physically meaningful interval between zero and one. The panel temperature is calculated using a one-node lumped thermal model:

The first term on the right-hand side represents absorbed solar energy. The second term represents energy exported as electrical power, while the third term represents net radiative heat loss to deep space. Their difference defines the instantaneous net heating rate:

The transient temperature is obtained by integrating

from the specified initial temperature. For example, a first-order time update can be expressed as

A higher-order ordinary differential equation integrator may be used when improved accuracy or numerical stability is required. The thermal capacity  controls the response rate, with a larger value producing a slower temperature response.

Under constant illumination and attitude conditions, the thermal equilibrium temperature is obtained by setting the temperature derivative to zero:

Because this equation contains both a linear temperature-dependent efficiency term and a fourth-order radiation term, \(T_{\mathrm{eq}}\) is normally determined using a numerical root solver. If the conversion efficiency is treated as constant, an explicit approximation is available:

The transient temperature generally lags behind the instantaneous equilibrium temperature because of the panel’s finite thermal capacity. The current single-node model neglects structural conduction, Earth infrared radiation, albedo, internal heat dissipation, battery losses, and maximum-power-point-tracking dynamics.

Access Calculation Using Cubic Hermite Interpolation

The orbit propagator provides the satellite position and velocity at discrete epochs. Consider two adjacent states, and  defined at tk and tk+1, respectively. The interval duration and normalized interpolation variable are defined as

Within this interval, the satellite position is represented by a cubic polynomial

Differentiating the polynomial with respect to time gives the interpolated velocity:

The interpolant must reproduce both the position and velocity at the two endpoints:

Substitution of these constraints into the cubic polynomial gives

Solving these four endpoint equations gives the following remaining polynomial coefficients:

The resulting position interpolation can therefore be written in the standard cubic Hermite form:

where the Hermite basis functions are

The interpolated inertial position is subsequently transformed into the Earth-centered, Earth-fixed frame:where  is the time-dependent transformation matrix. If the ground-station position is, the station-to-satellite line-of-sight vector is

For a ground station at geodetic latitude  and longitude, the line-of-sight vector is transformed into the local east-north-up frame:

Because  is the projection along the local zenith direction, the elevation angle satisfies

For improved numerical robustness near the horizon, the elevation is evaluated as

The visibility margin is defined as

where  is the minimum permitted elevation angle. The binary visibility state is consequently

A visibility boundary exists in an interval  when  The boundary epoch  is obtained by solving  A secant estimate of the boundary time can first be calculated from

Brent’s method then combines secant interpolation, inverse quadratic interpolation, and bisection to refine the root. The iteration terminates when where  is the required time tolerance. A negative-to-positive crossing of  represents Acquisition of Signal (AOS), whereas a positive-to-negative crossing represents Loss of Signal (LOS).

This method determines access boundaries without repeating the orbital propagation during root refinement. It can therefore achieve timing accuracy finer than the original ephemeris sampling interval at a relatively low computational cost. The ephemeris interval must remain sufficiently short to prevent an entire visibility window from occurring between two samples without a detectable sign change. Strictly, this implementation uses piecewise cubic Hermite interpolation because both position and velocity are constrained at every node. The resulting trajectory is continuously differentiable, although second-derivative continuity is not generally guaranteed.

Detailed Functional Design

Common data models

Table 6  Core data objects

| Object | Stored data | Invariant or behavior |
| --- | --- | --- |
| OrbitState | epoch, position_m[3], velocity_m_s[3], frame | Aware UTC; SI; frame is GCRS, ITRS, or TEME. |
| Ephemeris | times[N], positions_m[N,3], velocities_m_s[N,3], frame | Aligned lengths; monotonic time; state(i) reconstruction. |
| ClassicalElements | a, e, i, Ω, ω, ν, optional epoch | Elliptic osculating elements; radians and metres. |
| AttitudeEphemeris | times_s, quaternion xyzw, body rates | Body-to-inertial active quaternion; rates in body frame. |
| ThermalPowerResult | temperature, electrical power, incident power | Arrays aligned to input elapsed times. |
| AccessWindow | start, end | Positive duration property. |
| AccessResult | windows plus sampled margins and geometry | Provides traceable event-search diagnostics. |

Orbit propagation

Table 7  Propagator selection and model semantics

| Propagator | Model | Frame | Primary use |
| --- | --- | --- | --- |
| TwoBodyPropagator | Exact elliptic Kepler solution from the initial osculating state | GCRS | Baseline, initialization, unit tests, short arcs |
| J2Propagator | Central gravity + J2, integrated by DOP853 | GCRS | Dominant node/perigee drift and a light numerical baseline |
| SGP4Propagator | Vallado SGP4/SDP4 applied to TLE mean elements | TEME | Catalog/TLE propagation and operational screening |
| HighPrecisionPropagator | Cowell integration with composable force stack | GCRS | Engineering high-fidelity prediction and sensitivity studies |

Two-body propagator

TwoBodyPropagator converts the initial Cartesian state to elliptic elements, advances the mean anomaly with the Keplerian mean motion, solves Kepler's equation, and converts the result back to Cartesian position and velocity. It is analytical and does not accumulate numerical integration error over a single ideal orbit, but it deliberately omits all perturbations.

J2 propagator

J2Propagator composes central gravity and the standard J2 acceleration and integrates the six-state Cartesian equations with SciPy DOP853. Users configure relative tolerance, position and velocity absolute tolerances, and maximum integration step. It excludes atmosphere, third bodies, SRP, and higher harmonics.

SGP4 propagator

SGP4Propagator accepts two TLE lines, validates the epoch and satellite record through python-sgp4, and returns SI-unit TEME states. TLE age and element quality normally dominate the prediction error. The interface therefore keeps TEME explicit and requires transform_ephemeris for GCRS or ITRS use.

High-precision propagator

HighPrecisionPropagator uses adaptive DOP853 Cowell integration. The default force stack is central gravity plus EGM2008 70×70, Sun/Moon third-body gravity, and Schwarzschild correction. When area-to-mass is positive, the stack also enables segmented exponential-atmosphere drag and cannonball SRP. The Sun and Moon ephemerides are shared across forces using sampled interpolation. Additional objects implementing acceleration(elapsed_s, state, epoch) can be appended through extra_forces.

Table 8  HPOP configuration

| Parameter | Default | Design meaning |
| --- | --- | --- |
| gravity_degree / order | 70 / same as degree | Static harmonic truncation; None selects the J2 baseline path. |
| area_to_mass_m2_kg | 0.0 | Enables drag and SRP when positive. |
| drag_coefficient | 2.2 | Cannonball aerodynamic coefficient. |
| reflectivity_coefficient | 1.3 | Cannonball SRP coefficient. |
| include_third_body | True | Sun and Moon point-mass differential gravity. |
| include_relativity | True | Schwarzschild correction used by the engineering stack. |
| relative_tolerance | 1e-11 | DOP853 relative error control. |
| position / velocity tolerance | 1 mm / 1 μm·s⁻¹ | Component-wise absolute error scaling. |
| max_step_s | 120 s | Upper bound on adaptive integration step. |

High-order Earth gravity

GravityFieldCoefficients stores fully normalized static cosine/sine coefficient matrices, model GM, reference radius, tide system, and provenance. from_icgem reads static gfc records and supports explicit degree/order truncation. load_builtin_egm2008 verifies and loads the packaged 0–120 subset.

SphericalHarmonicGravity computes the non-central potential and acceleration in the fixed frame. It intentionally excludes the degree-zero central term because HPOP adds central gravity separately using the model's GM. EarthOrientationCache precomputes GCRS→ITRS rotations and interpolates them with SLERP; the default node interval is 300 s.

Table 9  Bundled EGM2008 data contract

| Attribute | Value |
| --- | --- |
| Packaged coverage | Degree and order 0–120 |
| HPOP default | 70×70 |
| Normalization | Fully normalized |
| Tide system | Tide-free |
| Model GM | 3.986004415 × 10¹⁴ m³/s² |
| Reference radius | 6,378,136.3 m |
| Subset SHA-256 | 6ae505fcd7acfed84c5f69ff3e729be9cbb450162fd6bcf9116584c5b00570f2 |
| External static models | ICGEM gfc records; GRACE monthly/static files after epoch reduction if required |

Model limitation: The loader does not directly evaluate time-variable gfct, trnd, asin, or acos records. A GRACE monthly solution or other time-variable field must first be reduced to static coefficients for the task epoch.

Element and Cartesian-state conversion

oe_to_rv converts elliptic osculating classical elements to an inertial Cartesian state. rv_to_oe performs the inverse conversion and applies fixed conventions in singular cases: argument of periapsis is zero for circular inclined orbits; RAAN is zero for equatorial eccentric orbits; both are zero for circular equatorial orbits. The returned anomaly then represents argument of latitude or true longitude as appropriate.

Coordinate and frame transformations

eci_to_ecef converts a GCRS OrbitState to ITRS; ecef_to_eci performs the inverse. Both transport velocity, including the rotational contribution. transform_ephemeris extends the same contract to sampled GCRS, ITRS, or TEME ephemerides through Astropy/ERFA. WGS-84 geodetic_to_ecef and ecef_to_geodetic support target definition and ground geometry.

Operational data: Astropy transformations depend on UT1−UTC, polar motion, and leap seconds. Production runs should archive the controlled IERS Bulletin A/B table used for the calculation.

Attitude conversion and dynamics

ypr_to_euler interprets yaw-pitch-roll as aerospace intrinsic Z-Y-X rotation and converts it to a requested Euler sequence. Upper-case sequences are intrinsic and lower-case sequences are extrinsic, matching SciPy Rotation. euler_to_dcm returns the active direction-cosine matrix.

EulerDynamics integrates the body-to-inertial quaternion in SciPy xyzw order and the rigid-body Euler equation. Inertia may be supplied as principal moments or a matrix. Torque is expressed in the body frame and may be constant or a callable of time, quaternion, and body rate.

q̇ = 1/2 · q ⊗ [ωx, ωy, ωz, 0],        Iω̇ = τ − ω × (Iω)

Solar environment and radiation pressure

Table 10  Solar functions

| Function | Method | Output |
| --- | --- | --- |
| sun_position_analytical | Low-order geocentric analytical Sun model; native MOD, optional GCRS conversion | Earth-to-Sun vector [m] |
| sun_position_ephemeris | Astropy built-in ephemeris or controlled local JPL kernel | GCRS-aligned Earth-to-Sun vector [m] |
| sun_beta_angle | Signed Sun elevation above the instantaneous orbit plane | rad or deg |
| eclipse_fraction | Apparent-disc overlap with conical umbra/penumbra geometry | Visible solar-disc fraction 0–1 |
| sun_intensity | Solar constant scaled by actual Sun distance and optional eclipse fraction | Direct irradiance [W/m²] |
| sun_radiation_pressure | Cannonball SRP, directed away from the Sun, optionally eclipsed | Acceleration [m/s²] |

The analytical and ephemeris Sun functions are separate because they serve different traceability and fidelity needs. The analytical model is small and deterministic. The ephemeris path can use Astropy's offline built-in solution or a task-controlled JPL binary kernel when jplephem is installed.

Power considering panel temperature

SolarPanel contains area, reference conversion efficiency, temperature coefficient, absorptivity, emissivity, radiating area, heat capacity, and deep-space sink temperature. power_considering_thermal integrates a one-node lumped thermal balance. Incidence cosine is clipped to [0,1]; irradiance is expected to include eclipse before entering the model.

The panel is isothermal. Absorbed solar energy raises the node temperature after exported electrical power is removed; radiation to deep space cools the panel. Structural conduction, Earth infrared, albedo, battery behavior, and MPPT dynamics are outside the current model.

Access propagation and interpolation

GroundTarget defines a fixed WGS-84 latitude, longitude, altitude, and name. NadirSensor defines a fixed VVLH boresight through pitch and roll, a half-cone field of view, minimum target elevation, and optional maximum range. Access is the intersection of the target local-horizon margin, sensor off-boresight margin, range margin, and Earth-clear geometry.

access_propagation first samples any compatible orbit propagator and then searches for events. access_interpolation receives an existing GCRS Ephemeris and uses cubic Hermite interpolation with the stored velocity derivatives. Both scan at scan_step_s and refine each sign-changing boundary with Brent's method to boundary_tolerance_s.

Table 11  Access outputs

| Output | Description |
| --- | --- |
| windows | Tuple of AccessWindow(start, end) objects with duration_s. |
| sample_times | Times used by the event scan. |
| combined_margin_rad | Minimum of all active geometric margins; positive means accessible. |
| elevation_rad | Target elevation in the local topocentric frame. |
| off_boresight_rad | Angle between sensor boresight and target line of sight. |
| slant_range_m | Spacecraft-to-target distance. |

Public Interface Design

General contracts

- Lengths are metres, velocities are metres per second, elapsed times are seconds, torque is N·m, temperature is kelvin, and power is watts.

- Angles are radians unless degrees=True or a from_degrees constructor is explicitly used.

- datetime values must be timezone-aware; UTC is the operational convention.

- Numerical propagation initial states must be GCRS; SGP4 outputs TEME; Earth-fixed calculations use ITRS.

- Functions validate dimensions, finite values, monotonic time arrays, physical parameter ranges, and compatible frames.

Primary calling interfaces

Table 12  Interface families

| Family | Primary call | Input | Output |
| --- | --- | --- | --- |
| Orbit | propagator.propagate(initial, times) / propagate_grid(...) | OrbitState + epochs | Ephemeris |
| Elements | oe_to_rv(elements); rv_to_oe(r,v) | ClassicalElements or Cartesian state | Cartesian state or elements |
| Gravity | from_icgem(...); acceleration_fixed(r) | Coefficient file + ITRS position | Coefficients / acceleration |
| Frames | eci_to_ecef(state); transform_ephemeris(eph, target) | State or ephemeris | Transformed state/ephemeris |
| Attitude | EulerDynamics(...).propagate(q0, ω0, times) | Inertia, torque, initial attitude | AttitudeEphemeris |
| Solar | Sun/beta/eclipse/intensity/SRP functions | Epoch, orbit state, area/mass | Vectors, angles, fractions, acceleration |
| Power | power_considering_thermal(times,I,cosθ,panel) | Time histories + SolarPanel | ThermalPowerResult |
| Access | access_propagation(...) / access_interpolation(...) | Orbit, target, sensor, event settings | AccessResult |

Minimal end-to-end call

from datetime import datetime, timedelta, timezone from ntu_space_dynamics import ( ClassicalElements, HighPrecisionPropagator, OrbitState, SolarPanel, oe_to_rv, sun_position_ephemeris, eclipse_fraction, sun_intensity, power_considering_thermal, ) epoch = datetime(2026, 1, 1, tzinfo=timezone.utc) oe = ClassicalElements.from_degrees( 7_000_000.0, 0.001, 97.5, 20.0, 0.0, 0.0, epoch ) r0, v0 = oe_to_rv(oe) initial = OrbitState(epoch, r0, v0, "GCRS") eph = HighPrecisionPropagator(gravity_degree=70).propagate_grid( initial, epoch + timedelta(hours=3), step_s=60.0 )

The full example in examples/end_to_end.py adds Sun geometry, illumination, thermal power, rigid-body attitude, interpolated access, summary statistics, and multi-format plots.

End-to-End Operational Workflow

- Create an aware UTC epoch and define the initial Cartesian state, classical elements, or TLE record.

- Select the propagation semantics: two-body, J2, SGP4, or a declared HPOP force stack.

- Generate one aligned ephemeris and transform frames only through the public coordinate interfaces.

- Evaluate Sun vectors, beta angle, eclipse fraction, irradiance, SRP, thermal power, and attitude at the same epochs.

- Evaluate access from the generated ephemeris, refining each open/close event boundary.

- Export numerical results, model metadata, and diagnostic plots for engineering review.

Figure 3  End-to-end propagation, solar, thermal-power, and attitude histories

The diagnostic figure is an engineering sanity-check surface rather than a formal orbit-determination validation product. It makes state discontinuities, eclipse transitions, unexpected power saturation, thermal divergence, quaternion normalization problems, and body-rate instability visible in one view.

Numerical Design and Performance

Table 13  Numerical methods and controls

| Area | Method | Primary controls |
| --- | --- | --- |
| Two-body | Analytical elliptic Kepler solution | Epoch grid and gravitational parameter |
| J2/HPOP | SciPy solve_ivp DOP853 | rtol, position/velocity atol, max_step_s |
| Attitude/thermal | Adaptive non-stiff integration | Input sampling and tolerance configuration |
| Earth orientation | Astropy/ERFA rotation nodes + SLERP | IERS data and sample_step_s |
| Sun/Moon | Shared sampled ephemeris interpolation | Ephemeris source and sampling interval |
| Access state | Cubic Hermite interpolation | Orbit sample spacing and velocity quality |
| Access events | Fixed scan + Brent root refinement | scan_step_s and boundary_tolerance_s |
| Gravity | Normalized harmonic summation | Coefficient source, degree, order, tide system |

High-order gravity is the main controllable cost in typical HPOP runs. Raising the degree/order increases both coefficient count and recurrence work. The recommended practice is to demonstrate convergence for the mission altitude and forecast duration, then select the lowest truncation that meets the error budget. Access scan_step_s must be smaller than the shortest window that must be detected.

Verification and Validation

The current baseline passes 23 unit tests under Python's unittest runner. Tests use analytical invariants, round trips, Vallado reference vectors, published gravity metadata, finite-difference checks, model degeneration, and event-boundary refinement. This is implementation verification; mission-specific validation still requires comparison with authoritative ephemerides, tracking data, or a trusted tool.

Table 14  Automated verification summary

| Test group | Count | Representative evidence |
| --- | --- | --- |
| Elements and propagators | 7 | OE/RV round trips including singular cases; two-body period closure; J2 energy; HPOP/J2 equivalence; Vallado SGP4 vector |
| Gravity | 8 | EGM metadata and zonals; C̄20→J2 reduction; finite-difference potential gradient; high-degree effect; ICGEM truncation; HPOP arc |
| Coordinates and solar | 4 | GCRS/ITRS position-velocity round trip; analytical/ephemeris Sun direction; eclipse; beta and SRP direction |
| Attitude, power, access | 4 | YPR rotation equivalence; torque-free invariants; thermal trend; refined access boundaries |

Executed baseline: 23 tests passed on 17 August 2026. The end-to-end example also generated PNG, PDF, SVG, and TIFF diagnostic figures.

Deployment and Configuration

Runtime

- Python 3.10 or later.

- Required: NumPy ≥1.24, SciPy ≥1.10, Astropy ≥5.3, Matplotlib ≥3.7, python-sgp4 ≥2.23.

- Optional: jplephem ≥2.18 for a controlled local JPL binary ephemeris kernel.

- Packaged data: EGM2008 tide-free static coefficients through degree/order 120 and SHA-256 sidecar.

Installation and VS Code interpreter

cd E:\MyProject\NTU_Space_Data_Center python -m venv .venv .\.venv\Scripts\python.exe -m pip install -e . .\.venv\Scripts\python.exe -m unittest discover -s tests -v

In Visual Studio Code select Python: Select Interpreter and choose E:\MyProject\NTU_Space_Data_Center\.venv\Scripts\python.exe. Production deployments should archive the exact coefficient file, IERS table, local ephemeris kernel if used, package versions, and model configuration alongside each result.

Limitations and Planned Evolution

Table 15  Present limitations and recommended extensions

| Subsystem | Current boundary | Recommended extension |
| --- | --- | --- |
| Gravity | Static fully normalized field; no tides or in-run time-variable coefficients | Solid/ocean/pole tides; task-epoch GRACE evaluation; EOP/data version manager |
| Atmosphere | Segmented exponential density; no space weather response | NRLMSISE-00/JB2008 with archived F10.7, F10.7a, and Ap/Kp |
| SRP | Cannonball area/mass and single reflectivity coefficient | Faceted geometry, self-shadowing, Earth albedo and IR |
| Ephemerides | Astropy built-in by default | Controlled DE440/DE441 or SPICE kernel bundle |
| Thermal/power | One isothermal panel node; no conduction or electrical storage/control | Multi-node thermal network, Earth IR/albedo, battery, EPS, MPPT |
| Attitude | Rigid body with external torque; no actuator/sensor models | Reaction wheels, magnetorquers, disturbances, guidance and control loops |
| Access | Fixed target and fixed VVLH sensor; no terrain/refraction/cloud/maneuver time | Terrain horizon, atmosphere, keep-out constraints, slew and payload modes |
| Validation | Unit tests and engineering sanity plots | Cross-tool campaigns and mission-tracking residual analysis |

Requirements Traceability Matrix

The matrix maps every requested capability to its implemented interface, computational model, primary output, and verification status.

Table 16  Functional requirements response

| ID | Requested capability | Implemented interface | Design model | Primary output | Status |
| --- | --- | --- | --- | --- | --- |
| F-01 | Twobody | TwoBodyPropagator | Elliptic Kepler analytical propagation | GCRS Ephemeris | Implemented / tested |
| F-02 | J2 | J2Propagator | Central gravity + J2, DOP853 | GCRS Ephemeris | Implemented / tested |
| F-03 | SGP4 | SGP4Propagator | Vallado SGP4/SDP4, TLE mean elements | TEME Ephemeris | Implemented / Vallado vector tested |
| F-04 | HPOP | HighPrecisionPropagator | Cowell + declared perturbation stack | GCRS Ephemeris | Implemented / baseline tested |
| F-05 | High-order gravity | GravityFieldCoefficients; SphericalHarmonicGravity | EGM2008 70×70 default, packaged to 120; external static ICGEM/GRACE | Potential / acceleration / HPOP | Implemented / 8 gravity tests |
| F-06 | OE2RV | oe_to_rv | Elliptic osculating elements to Cartesian state | r [m], v [m/s] | Implemented / round-trip tested |
| F-07 | RV2OE | rv_to_oe | Cartesian state to elements with singular conventions | ClassicalElements | Implemented / singular cases tested |
| F-08 | ECI2ECEF | eci_to_ecef; ecef_to_eci; transform_ephemeris | GCRS↔ITRS and TEME transforms with velocity | OrbitState / Ephemeris | Implemented / round-trip tested |
| F-09 | YPR2EULER | ypr_to_euler; euler_to_dcm | Intrinsic Z-Y-X YPR and arbitrary Euler sequence | Euler angles / DCM | Implemented / rotation tested |
| F-10 | EULERDYNAMICS | EulerDynamics | Quaternion kinematics + rigid-body Euler equation | AttitudeEphemeris | Implemented / invariants tested |
| F-11 | SUN POSITION Analytical | sun_position_analytical | Low-order MOD model with optional GCRS conversion | Sun vector [m] | Implemented / direction tested |
| F-12 | SUN POSITION ephemeris | sun_position_ephemeris | Astropy built-in or local JPL kernel | Sun vector [m] | Implemented / direction tested |
| F-13 | SUN BETA ANGLE | sun_beta_angle | Signed Sun elevation above orbit plane | Angle | Implemented / sign tested |
| F-14 | SUN INTENSITY | eclipse_fraction; sun_intensity | Conical eclipse and Sun-distance correction | 0–1 fraction; W/m² | Implemented / eclipse tested |
| F-15 | Power considering thermal | SolarPanel; power_considering_thermal | One-node energy balance and temperature-dependent efficiency | ThermalPowerResult | Implemented / thermal trend tested |
| F-16 | Sun radiation pressure | sun_radiation_pressure | Eclipsed cannonball SRP | Acceleration [m/s²] | Implemented / direction tested |
| F-17 | ACCESS PROPAGATION | access_propagation | Propagate then scan/refine target/sensor geometry | AccessResult | Implemented / event tested |
| F-18 | ACCESS INTERPOLATION | access_interpolation | Cubic Hermite state interpolation + Brent refinement | AccessResult | Implemented / boundary tested |
| F-19 | End-to-end diagnostics | examples/end_to_end.py | Aligned orbit, solar, power, thermal, attitude, access and plots | Statistics + PNG/PDF/SVG/TIFF | Implemented / executed |

Appendix A — Public API Inventory

Table A-1  Exported names in ntu_space_dynamics.__all__

| Public name | Kind | Contract summary |
| --- | --- | --- |
| AccessResult | Type | Access windows and sampled geometry/margins. |
| AccessWindow | Type | Start/end UTC and derived duration. |
| AttitudeEphemeris | Type | Quaternion and body-rate time history. |
| ClassicalElements | Type | Elliptic osculating elements. |
| Ephemeris | Type | Sampled orbit positions and velocities. |
| EulerDynamics | Class | Rigid-body attitude propagation. |
| EarthOrientationCache | Class | Cached GCRS→ITRS rotations with SLERP. |
| GroundTarget | Type | Fixed WGS-84 ground point. |
| GravityFieldCoefficients | Type | Static fully normalized harmonic coefficients. |
| HighPrecisionPropagator | Class | Composable Cowell HPOP. |
| J2Propagator | Class | Central + J2 numerical propagation. |
| NadirSensor | Type | Fixed VVLH sensor and access constraints. |
| OrbitState | Type | Single-epoch SI Cartesian state. |
| SGP4Propagator | Class | TLE propagation to TEME. |
| SolarPanel | Type | Thermal/electrical panel parameters. |
| SphericalHarmonicGravity | Class | Static non-central gravity synthesis. |
| ThermalPowerResult | Type | Temperature and power histories. |
| TwoBodyPropagator | Class | Exact elliptic Kepler propagation. |
| access_interpolation | Function | Access from a sampled GCRS ephemeris. |
| access_propagation | Function | Propagate then compute access. |
| ecef_to_eci | Function | ITRS/ECEF to GCRS state. |
| ecef_to_geodetic | Function | WGS-84 ECEF to latitude/longitude/altitude. |
| eci_to_ecef | Function | GCRS to ITRS/ECEF state. |
| eclipse_fraction | Function | Visible solar-disc fraction. |
| euler_to_dcm | Function | Euler angles to active DCM. |
| geodetic_to_ecef | Function | WGS-84 latitude/longitude/altitude to ECEF. |
| load_builtin_egm2008 | Function | Load packaged EGM2008 to degree 120. |
| oe_to_rv | Function | Elements to Cartesian state. |
| power_considering_thermal | Function | Panel temperature and electrical power. |
| rv_to_oe | Function | Cartesian state to classical elements. |
| sun_beta_angle | Function | Signed Sun/orbit-plane angle. |
| sun_intensity | Function | Direct irradiance with optional eclipse. |
| sun_position_analytical | Function | Analytical Sun vector. |
| sun_position_ephemeris | Function | Ephemeris Sun vector. |
| sun_radiation_pressure | Function | Cannonball SRP acceleration. |
| transform_ephemeris | Function | GCRS/ITRS/TEME sampled state transform. |
| ypr_to_euler | Function | Aerospace YPR to arbitrary Euler sequence. |

Appendix B — Model-Selection Guidance

Table B-1  Recommended model selection

| Question | Recommended starting point |
| --- | --- |
| Need a clean algorithm baseline or short initialization? | TwoBodyPropagator. |
| Need the dominant oblateness drift with a small force stack? | J2Propagator. |
| Starting from a TLE or catalog product? | SGP4Propagator; keep TEME explicit. |
| Need perturbation composition and high-order gravity? | HighPrecisionPropagator with declared coefficients, tolerances, and environmental inputs. |
| Need fast repeated access on an existing orbit? | access_interpolation with sufficiently dense samples. |
| Need access while selecting or comparing propagators? | access_propagation. |
| Need deterministic offline Sun calculations? | Astropy built-in ephemeris or the analytical model, depending on the error budget. |
| Need higher solar-system accuracy? | Local controlled JPL kernel with jplephem. |

Appendix C — References

Pavlis, N. K. et al. (2012). The development and evaluation of the Earth Gravitational Model 2008 (EGM2008). Journal of Geophysical Research, 117, B04406.

Vallado, D. A. et al. (2006). Revisiting Spacetrack Report #3. AIAA/AAS Astrodynamics Specialist Conference.

IERS Conventions (2010), IERS Technical Note 36.

Montenbruck, O. and Gill, E. (2000). Satellite Orbits: Models, Methods and Applications.

Astropy, ERFA, SciPy, NumPy, Matplotlib, and python-sgp4 project documentation.

NTU Space Dynamics source documentation: README.md, docs/model-fidelity.md, docs/open-source-evaluation.md, and data/gravity/README.md.